RESL Thermal Battery Agent

Report generated on 9 April 2026 at 05:42 pm

Query: Data of qualified battery code 44 Review data of battery code 44. The determination of discharge duration shall be performed for all builds. The procedure is outlined in the rulebook. Note the temperature of discharge. Record the maximum open circuit voltage. Record the maximum on load voltage. Record the activation time. This should be done for all the builds. For all builds that have met the minimum discharge duration create a table-1 which gives the build number, temperature of discharge, condition of discharge ( static, vibration, acceleration ), discharge duration, maximum open circuit voltage, maximum on load voltage and activation time for that build. Performance Summary of battery code 44 Create a table-2 where you provide a summary of the data of batteries that have been tested under the exact same condition. This data is subset of table-1 and the information in the table-2 should be derived from table-1. Table-2 should contain the discharge temperature, discharge type, number of batteries, minimum duration, maximum duration, median duration, maximum on load voltage, minimum on load voltage, maximum open circuit voltage, minimum circuit voltage. In populating this table no data should be included where the standard discharge duration has not been met. Performance Degradation Analysis for battery code 44 Create table-3 which shows Performance Degradation Ratio. The table shall have the following columns: Temperature of Discharge, Discharge Type, Degradation Performance Ratio. The Discharge Type has been defined in the rulebook para 2.2. The method of computing Performance Degradation Ratio is given in the para 3.1 in the rulebook. Using that criteria specified there provide a table-3 as defined above. The comparative data is only for batteries that have met Minimum Discharge Duration. Create table-4 which gives information on Temperature Degradation Ratio. Method of computation is defined in para 3.2 in the rulebook. The columns in the table shall be Type of Discharge, Temperature of Discharge, Temperature Degradation Ratio. The comparative data is only for batteries that have met Minimum Discharge Duration. In addition to the above create table-5 for each build of battery type 44 which has met the minimum discharge duration only. Table-5 shall be numbered as 5.1, 5.2, 5.3 etc one table for each build. The table shall be as per format given in rule 4.6.1. The rules for filling this table are given in rule book para 4.1 to para 5.0.Calculate the anode active material weight in grams and cathode active material weight in grams for battery 44 for all builds using Rule 4.3 and Rule 4.4.for battery code 44 5.0 Present table-6.1 and 6.2 having the information given below. This is to be presented for all builds that have met the minimum discharge duration. Table 6.1 Build ID Anode Weight per Electrode (4.3.1) Number of Parallel Stack(4.3.2) Total Anode Material (4.3.3) Amount of LiSi (4.3.4) Table 6.2 Build ID Cathode Weight per Electrode (4.4.1) Number of Parallel Stack(4.4.2) Total Cathode Material (4.4.3) Amount of FeS2(4.4.4). calculate calorific value per gram per second for battery code 44 of all builds. calculate calorific value per gram per second for battery code 44 of all builds. for Battery type 44 for all Builds whether the battery has met "Minimum Discharge Duration" or not determined the contents of the table given in rule book clause 9.1 by executing the rule book clause 7.1 to clause 9.7. And give me the graphs or charts based on the clauses 10 and 11 and 12

# Comprehensive Analysis Report — Battery 44

26 builds analyzed: 108, 109, 112, 113, 116, 117, 119, 120, 121, 123, 124, 125, 126, 127, 128, 135, 136, 137, 138, 139, 140, 141, 142, 143, 144, 146

---

## Table 1: Performance Summary of Qualified Builds

Builds that met the minimum discharge duration

| Build | Temp (°C) | Condition | Duration (s) | Max OCV (V) | Max On-Load (V) | Activation Time (ms) |
| --- | --- | --- | --- | --- | --- | --- |
| 108 | +55 | static | 632.36 | 39.77 | 39.03 | 294.1 |
| 113 | -10 | static | 594.63 | 38.8 | 38.46 | 296.2 |
| 116 | -10 | static | 526.35 | 39.38 | 39.21 | 359.1 |
| 117 | +55 | static | 584.73 | 39.46 | 39.17 | 239.1 |
| 119 | +55 | RV | 555.09 | 39.53 | 39.36 | 257.9 |
| 120 | +55 | sttatic | 656.85 | 39.68 | 39.39 | 268.1 |
| 121 | -10 | static | 564.81 | 39.15 | 38.97 | 346.4 |
| 123 | -10 | static | 516.91 | 39.85 | 39.64 | 366.2 |
| 124 | +55 | static | 598.12 | 40.12 | 39.87 | 252.4 |
| 126 | +55 | acceleration | 465.81 | 40.05 | 39.78 | 240.5 |
| 127 | -10 | RV | 468.63 | 39.42 | 38.78 | 309.6 |
| 128 | -10 | acceleration | 453.35 | 40.32 | 39.69 | 600.2 |
| 136 | -10 | static | 573.1 | 38.48 | 38.24 | 246.8 |
| 138 | -10 | static | 502.94 | 38.06 | 37.8 | 294.4 |
| 139 | +55 | static | 499.69 | 38.43 | 38.12 | 263.7 |
| 140 | -10 | static | 523.31 | 37.83 | 37.8 | 278.5 |
| 142 | -10 | acceleration | 482.59 | 38.28 | 38.07 | 238.0 |
| 143 | +55 | static | 588.84 | 38.41 | 38.31 | 211.2 |
| 146 | -10 | static | 585.12 | 39.38 | 39.13 | 233.3 |

✅ 19 builds PASSED: 108, 113, 116, 117, 119, 120, 121, 123, 124, 126, 127, 128, 136, 138, 139, 140, 142, 143, 146

❌ 7 builds FAILED: 109, 112, 125, 135, 137, 141, 144

---

## Table 2: Performance Summary by Discharge Condition

Aggregated data for qualified builds tested under identical conditions

| Temp (°C) | Type | Count | Min Duration (s) | Max Duration (s) | Median Duration (s) | Max On-Load (V) | Min On-Load (V) | Max OCV (V) | Min OCV (V) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| +55 | RV | 1 | 555.09 | 555.09 | 555.09 | 39.36 | 39.36 | 39.53 | 39.53 |
| +55 | acceleration | 1 | 465.81 | 465.81 | 465.81 | 39.78 | 39.78 | 40.05 | 40.05 |
| +55 | static | 5 | 499.69 | 632.36 | 588.84 | 39.87 | 38.12 | 40.12 | 38.41 |
| +55 | sttatic | 1 | 656.85 | 656.85 | 656.85 | 39.39 | 39.39 | 39.68 | 39.68 |
| -10 | RV | 1 | 468.63 | 468.63 | 468.63 | 38.78 | 38.78 | 39.42 | 39.42 |
| -10 | acceleration | 2 | 453.35 | 482.59 | 467.97 | 39.69 | 38.07 | 40.32 | 38.28 |
| -10 | static | 8 | 502.94 | 594.63 | 545.58 | 39.64 | 37.8 | 39.85 | 37.83 |

---

## Table 3: Performance Degradation Ratio (Rule 3.1)

Ratio of dynamic condition median to static median at same temperature

| Temp (°C) | Discharge Type | Degradation Ratio |
| --- | --- | --- |
| +55 | RV | 0.943 |
| +55 | acceleration | 0.791 |
| +55 | sttatic | 1.115 |
| -10 | RV | 0.859 |
| -10 | acceleration | 0.858 |

---

## Table 4: Temperature Degradation Ratio (Rule 3.2)

Ratio of cold-temperature median to warm-temperature median for same discharge type

| Type | Temp (°C) | Degradation Ratio |
| --- | --- | --- |
| RV | -10 | 0.844 |
| acceleration | -10 | 1.005 |
| static | -10 | 0.927 |

---

## Tables 5, 6.1, 6.2: Active Material Utilization (Rules 4.3-4.7)

Processing 19 qualified builds

### Table 5: Build 121

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1034.929 | 1796.7517 | 782.2593 | static | -10 |
| 2 | 1.3 | 26.0 | 920.1289 | 1597.446 | 695.4867 | static | -10 |
| 3 | 1.4 | 28.0 | 799.4474 | 1387.9295 | 604.2686 | static | -10 |
| 4 | 1.5 | 30.0 | 671.6689 | 1166.0918 | 507.6862 | static | -10 |
| 5 | 1.6 | 32.0 | 537.8419 | 933.7533 | 406.532 | static | -10 |

### Table 5: Build 146

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1176.255 | 2042.1094 | 889.0816 | static | -10 |
| 2 | 1.3 | 26.0 | 1096.7926 | 1904.1538 | 829.0193 | static | -10 |
| 3 | 1.4 | 28.0 | 1013.8611 | 1760.1755 | 766.3349 | static | -10 |
| 4 | 1.5 | 30.0 | 880.5714 | 1528.7698 | 665.5868 | static | -10 |
| 5 | 1.6 | 32.0 | 628.9896 | 1091.9958 | 475.4268 | static | -10 |

### Table 5: Build 123

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1047.8731 | 1871.202 | 792.0432 | static | -10 |
| 2 | 1.3 | 26.0 | 992.6273 | 1772.5487 | 750.2852 | static | -10 |
| 3 | 1.4 | 28.0 | 922.0028 | 1646.4336 | 696.9031 | static | -10 |
| 4 | 1.5 | 30.0 | 814.6308 | 1454.6979 | 615.7451 | static | -10 |
| 5 | 1.6 | 32.0 | 593.4171 | 1059.6734 | 448.539 | static | -10 |

### Table 5: Build 117

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1192.1597 | 2257.8782 | 921.5829 | static | +55 |
| 2 | 1.3 | 26.0 | 1021.9253 | 1935.4646 | 789.9855 | static | +55 |
| 3 | 1.4 | 28.0 | 917.5854 | 1737.8511 | 709.327 | static | +55 |
| 4 | 1.5 | 30.0 | 764.3768 | 1447.6833 | 590.8912 | static | +55 |
| 5 | 1.6 | 32.0 | 601.6418 | 1139.4731 | 465.0911 | static | +55 |

### Table 5: Build 128

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 902.9985 | 1612.4973 | 682.5385 | acceleration | -10 |
| 2 | 1.3 | 26.0 | 850.8161 | 1519.3145 | 643.0961 | acceleration | -10 |
| 3 | 1.4 | 28.0 | 792.6398 | 1415.4282 | 599.1231 | acceleration | -10 |
| 4 | 1.5 | 30.0 | 731.1741 | 1305.668 | 552.6637 | acceleration | -10 |
| 5 | 1.6 | 32.0 | 656.8669 | 1172.9766 | 496.498 | acceleration | -10 |

### Table 5: Build 136

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1140.8423 | 1980.629 | 862.3147 | static | -10 |
| 2 | 1.3 | 26.0 | 1026.278 | 1781.7326 | 775.7203 | static | -10 |
| 3 | 1.4 | 28.0 | 897.3533 | 1557.905 | 678.2716 | static | -10 |
| 4 | 1.5 | 30.0 | 721.0384 | 1251.8028 | 545.0026 | static | -10 |
| 5 | 1.6 | 32.0 | 501.4368 | 870.55 | 379.015 | static | -10 |

### Table 5: Build 124

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1236.2504 | 2207.59 | 924.1612 | static | +55 |
| 2 | 1.3 | 26.0 | 1183.991 | 2114.2696 | 885.0946 | static | +55 |
| 3 | 1.4 | 28.0 | 1115.1081 | 1991.2645 | 833.601 | static | +55 |
| 4 | 1.5 | 30.0 | 996.0082 | 1778.5861 | 744.5677 | static | +55 |
| 5 | 1.6 | 32.0 | 720.9602 | 1287.4289 | 538.9551 | static | +55 |

### Table 5: Build 120

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1348.4582 | 2341.0733 | 1019.2428 | sttatic | +55 |
| 2 | 1.3 | 26.0 | 1252.9403 | 2175.2436 | 947.0448 | sttatic | +55 |
| 3 | 1.4 | 28.0 | 1081.0263 | 1876.7818 | 817.1023 | sttatic | +55 |
| 4 | 1.5 | 30.0 | 888.2178 | 1542.0448 | 671.3664 | sttatic | +55 |
| 5 | 1.6 | 32.0 | 629.7038 | 1093.2358 | 475.9666 | sttatic | +55 |

### Table 5: Build 139

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1034.1547 | 1795.4075 | 781.674 | static | +55 |
| 2 | 1.3 | 26.0 | 886.1616 | 1538.475 | 669.8122 | static | +55 |
| 3 | 1.4 | 28.0 | 761.1055 | 1321.3637 | 575.2876 | static | +55 |
| 4 | 1.5 | 30.0 | 634.0775 | 1100.829 | 479.2725 | static | +55 |
| 5 | 1.6 | 32.0 | 485.5575 | 842.9818 | 367.0125 | static | +55 |

### Table 5: Build 119

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1144.4676 | 1986.9229 | 865.0549 | RV | +55 |
| 2 | 1.3 | 26.0 | 1088.9879 | 1890.604 | 823.1201 | RV | +55 |
| 3 | 1.4 | 28.0 | 1008.0949 | 1750.1648 | 761.9765 | RV | +55 |
| 4 | 1.5 | 30.0 | 884.2091 | 1535.0852 | 668.3364 | RV | +55 |
| 5 | 1.6 | 32.0 | 683.9074 | 1187.3392 | 516.9368 | RV | +55 |

### Table 5: Build 127

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 945.5325 | 1688.4509 | 714.6882 | RV | -10 |
| 2 | 1.3 | 26.0 | 893.5717 | 1595.6637 | 675.4132 | RV | -10 |
| 3 | 1.4 | 28.0 | 814.1897 | 1453.9102 | 615.4117 | RV | -10 |
| 4 | 1.5 | 30.0 | 709.5522 | 1267.0575 | 536.3206 | RV | -10 |
| 5 | 1.6 | 32.0 | 549.3842 | 981.0432 | 415.2564 | RV | -10 |

### Table 5: Build 113

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1144.7112 | 2044.1271 | 874.9608 | static | -10 |
| 2 | 1.3 | 26.0 | 1011.2336 | 1805.7743 | 772.9371 | static | -10 |
| 3 | 1.4 | 28.0 | 784.8651 | 1401.5448 | 599.9122 | static | -10 |
| 4 | 1.5 | 30.0 | 584.943 | 1044.5411 | 447.1016 | static | -10 |
| 5 | 1.6 | 32.0 | 491.1146 | 876.9904 | 375.3838 | static | -10 |

### Table 5: Build 116

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1035.9536 | 1962.0333 | 800.8299 | static | -10 |
| 2 | 1.3 | 26.0 | 954.1124 | 1807.0311 | 737.5637 | static | -10 |
| 3 | 1.4 | 28.0 | 884.3663 | 1674.9362 | 683.6474 | static | -10 |
| 4 | 1.5 | 30.0 | 734.6895 | 1391.4574 | 567.9418 | static | -10 |
| 5 | 1.6 | 32.0 | 515.3078 | 975.9617 | 398.3517 | static | -10 |

### Table 5: Build 138

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 998.4082 | 1733.3476 | 754.6547 | static | -10 |
| 2 | 1.3 | 26.0 | 902.681 | 1567.1545 | 682.2986 | static | -10 |
| 3 | 1.4 | 28.0 | 786.1354 | 1364.8184 | 594.2067 | static | -10 |
| 4 | 1.5 | 30.0 | 656.3404 | 1139.4799 | 496.1001 | static | -10 |
| 5 | 1.6 | 32.0 | 521.4444 | 905.2854 | 394.1379 | static | -10 |

### Table 5: Build 143

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1216.1428 | 2111.359 | 919.2311 | static | +55 |
| 2 | 1.3 | 26.0 | 1059.2012 | 1838.891 | 800.6056 | static | +55 |
| 3 | 1.4 | 28.0 | 869.7995 | 1510.0686 | 657.4448 | static | +55 |
| 4 | 1.5 | 30.0 | 647.3766 | 1123.9177 | 489.3247 | static | +55 |
| 5 | 1.6 | 32.0 | 489.7894 | 850.3288 | 370.2112 | static | +55 |

### Table 5: Build 108

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1282.3752 | 2428.7409 | 991.3228 | static | +55 |
| 2 | 1.3 | 26.0 | 1170.343 | 2216.5587 | 904.7178 | static | +55 |
| 3 | 1.4 | 28.0 | 981.7913 | 1859.4532 | 758.9605 | static | +55 |
| 4 | 1.5 | 30.0 | 769.1706 | 1456.7625 | 594.5969 | static | +55 |
| 5 | 1.6 | 32.0 | 571.9735 | 1083.2831 | 442.1564 | static | +55 |

### Table 5: Build 126

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 958.5152 | 1711.6343 | 716.5397 | acceleration | +55 |
| 2 | 1.3 | 26.0 | 906.5281 | 1618.8002 | 677.6767 | acceleration | +55 |
| 3 | 1.4 | 28.0 | 865.6762 | 1545.8504 | 647.1378 | acceleration | +55 |
| 4 | 1.5 | 30.0 | 800.7958 | 1429.9925 | 598.6363 | acceleration | +55 |
| 5 | 1.6 | 32.0 | 591.9614 | 1057.0739 | 442.5218 | acceleration | +55 |

### Table 5: Build 142

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 962.6837 | 1671.3259 | 727.6521 | acceleration | -10 |
| 2 | 1.3 | 26.0 | 888.7383 | 1542.9484 | 671.7599 | acceleration | -10 |
| 3 | 1.4 | 28.0 | 846.6763 | 1469.9241 | 639.967 | acceleration | -10 |
| 4 | 1.5 | 30.0 | 685.282 | 1189.7257 | 517.9758 | acceleration | -10 |
| 5 | 1.6 | 32.0 | 544.0661 | 944.5592 | 411.2367 | acceleration | -10 |

### Table 5: Build 140

| S.No | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.2 | 24.0 | 1039.3977 | 1804.5099 | 785.637 | static | -10 |
| 2 | 1.3 | 26.0 | 957.4538 | 1662.2462 | 723.699 | static | -10 |
| 3 | 1.4 | 28.0 | 868.163 | 1507.2274 | 656.2079 | static | -10 |
| 4 | 1.5 | 30.0 | 722.5093 | 1254.3564 | 546.1144 | static | -10 |
| 5 | 1.6 | 32.0 | 536.7567 | 931.8693 | 405.7118 | static | -10 |

## Table 6.1: Anode Active Material (LiSi) — Battery 44

| Build | V/Cell | V Battery | Ampere-Seconds | As/g LiSi | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- |
| 121 | 1.2 | 24.0 | 1034.929 | 1796.7517 | static | -10 |
| 121 | 1.3 | 26.0 | 920.1289 | 1597.446 | static | -10 |
| 121 | 1.4 | 28.0 | 799.4474 | 1387.9295 | static | -10 |
| 121 | 1.5 | 30.0 | 671.6689 | 1166.0918 | static | -10 |
| 121 | 1.6 | 32.0 | 537.8419 | 933.7533 | static | -10 |
| 146 | 1.2 | 24.0 | 1176.255 | 2042.1094 | static | -10 |
| 146 | 1.3 | 26.0 | 1096.7926 | 1904.1538 | static | -10 |
| 146 | 1.4 | 28.0 | 1013.8611 | 1760.1755 | static | -10 |
| 146 | 1.5 | 30.0 | 880.5714 | 1528.7698 | static | -10 |
| 146 | 1.6 | 32.0 | 628.9896 | 1091.9958 | static | -10 |
| 123 | 1.2 | 24.0 | 1047.8731 | 1871.202 | static | -10 |
| 123 | 1.3 | 26.0 | 992.6273 | 1772.5487 | static | -10 |
| 123 | 1.4 | 28.0 | 922.0028 | 1646.4336 | static | -10 |
| 123 | 1.5 | 30.0 | 814.6308 | 1454.6979 | static | -10 |
| 123 | 1.6 | 32.0 | 593.4171 | 1059.6734 | static | -10 |
| 117 | 1.2 | 24.0 | 1192.1597 | 2257.8782 | static | +55 |
| 117 | 1.3 | 26.0 | 1021.9253 | 1935.4646 | static | +55 |
| 117 | 1.4 | 28.0 | 917.5854 | 1737.8511 | static | +55 |
| 117 | 1.5 | 30.0 | 764.3768 | 1447.6833 | static | +55 |
| 117 | 1.6 | 32.0 | 601.6418 | 1139.4731 | static | +55 |
| 128 | 1.2 | 24.0 | 902.9985 | 1612.4973 | acceleration | -10 |
| 128 | 1.3 | 26.0 | 850.8161 | 1519.3145 | acceleration | -10 |
| 128 | 1.4 | 28.0 | 792.6398 | 1415.4282 | acceleration | -10 |
| 128 | 1.5 | 30.0 | 731.1741 | 1305.668 | acceleration | -10 |
| 128 | 1.6 | 32.0 | 656.8669 | 1172.9766 | acceleration | -10 |
| 136 | 1.2 | 24.0 | 1140.8423 | 1980.629 | static | -10 |
| 136 | 1.3 | 26.0 | 1026.278 | 1781.7326 | static | -10 |
| 136 | 1.4 | 28.0 | 897.3533 | 1557.905 | static | -10 |
| 136 | 1.5 | 30.0 | 721.0384 | 1251.8028 | static | -10 |
| 136 | 1.6 | 32.0 | 501.4368 | 870.55 | static | -10 |
| 124 | 1.2 | 24.0 | 1236.2504 | 2207.59 | static | +55 |
| 124 | 1.3 | 26.0 | 1183.991 | 2114.2696 | static | +55 |
| 124 | 1.4 | 28.0 | 1115.1081 | 1991.2645 | static | +55 |
| 124 | 1.5 | 30.0 | 996.0082 | 1778.5861 | static | +55 |
| 124 | 1.6 | 32.0 | 720.9602 | 1287.4289 | static | +55 |
| 120 | 1.2 | 24.0 | 1348.4582 | 2341.0733 | sttatic | +55 |
| 120 | 1.3 | 26.0 | 1252.9403 | 2175.2436 | sttatic | +55 |
| 120 | 1.4 | 28.0 | 1081.0263 | 1876.7818 | sttatic | +55 |
| 120 | 1.5 | 30.0 | 888.2178 | 1542.0448 | sttatic | +55 |
| 120 | 1.6 | 32.0 | 629.7038 | 1093.2358 | sttatic | +55 |
| 139 | 1.2 | 24.0 | 1034.1547 | 1795.4075 | static | +55 |
| 139 | 1.3 | 26.0 | 886.1616 | 1538.475 | static | +55 |
| 139 | 1.4 | 28.0 | 761.1055 | 1321.3637 | static | +55 |
| 139 | 1.5 | 30.0 | 634.0775 | 1100.829 | static | +55 |
| 139 | 1.6 | 32.0 | 485.5575 | 842.9818 | static | +55 |
| 119 | 1.2 | 24.0 | 1144.4676 | 1986.9229 | RV | +55 |
| 119 | 1.3 | 26.0 | 1088.9879 | 1890.604 | RV | +55 |
| 119 | 1.4 | 28.0 | 1008.0949 | 1750.1648 | RV | +55 |
| 119 | 1.5 | 30.0 | 884.2091 | 1535.0852 | RV | +55 |
| 119 | 1.6 | 32.0 | 683.9074 | 1187.3392 | RV | +55 |
| 127 | 1.2 | 24.0 | 945.5325 | 1688.4509 | RV | -10 |
| 127 | 1.3 | 26.0 | 893.5717 | 1595.6637 | RV | -10 |
| 127 | 1.4 | 28.0 | 814.1897 | 1453.9102 | RV | -10 |
| 127 | 1.5 | 30.0 | 709.5522 | 1267.0575 | RV | -10 |
| 127 | 1.6 | 32.0 | 549.3842 | 981.0432 | RV | -10 |
| 113 | 1.2 | 24.0 | 1144.7112 | 2044.1271 | static | -10 |
| 113 | 1.3 | 26.0 | 1011.2336 | 1805.7743 | static | -10 |
| 113 | 1.4 | 28.0 | 784.8651 | 1401.5448 | static | -10 |
| 113 | 1.5 | 30.0 | 584.943 | 1044.5411 | static | -10 |
| 113 | 1.6 | 32.0 | 491.1146 | 876.9904 | static | -10 |
| 116 | 1.2 | 24.0 | 1035.9536 | 1962.0333 | static | -10 |
| 116 | 1.3 | 26.0 | 954.1124 | 1807.0311 | static | -10 |
| 116 | 1.4 | 28.0 | 884.3663 | 1674.9362 | static | -10 |
| 116 | 1.5 | 30.0 | 734.6895 | 1391.4574 | static | -10 |
| 116 | 1.6 | 32.0 | 515.3078 | 975.9617 | static | -10 |
| 138 | 1.2 | 24.0 | 998.4082 | 1733.3476 | static | -10 |
| 138 | 1.3 | 26.0 | 902.681 | 1567.1545 | static | -10 |
| 138 | 1.4 | 28.0 | 786.1354 | 1364.8184 | static | -10 |
| 138 | 1.5 | 30.0 | 656.3404 | 1139.4799 | static | -10 |
| 138 | 1.6 | 32.0 | 521.4444 | 905.2854 | static | -10 |
| 143 | 1.2 | 24.0 | 1216.1428 | 2111.359 | static | +55 |
| 143 | 1.3 | 26.0 | 1059.2012 | 1838.891 | static | +55 |
| 143 | 1.4 | 28.0 | 869.7995 | 1510.0686 | static | +55 |
| 143 | 1.5 | 30.0 | 647.3766 | 1123.9177 | static | +55 |
| 143 | 1.6 | 32.0 | 489.7894 | 850.3288 | static | +55 |
| 108 | 1.2 | 24.0 | 1282.3752 | 2428.7409 | static | +55 |
| 108 | 1.3 | 26.0 | 1170.343 | 2216.5587 | static | +55 |
| 108 | 1.4 | 28.0 | 981.7913 | 1859.4532 | static | +55 |
| 108 | 1.5 | 30.0 | 769.1706 | 1456.7625 | static | +55 |
| 108 | 1.6 | 32.0 | 571.9735 | 1083.2831 | static | +55 |
| 126 | 1.2 | 24.0 | 958.5152 | 1711.6343 | acceleration | +55 |
| 126 | 1.3 | 26.0 | 906.5281 | 1618.8002 | acceleration | +55 |
| 126 | 1.4 | 28.0 | 865.6762 | 1545.8504 | acceleration | +55 |
| 126 | 1.5 | 30.0 | 800.7958 | 1429.9925 | acceleration | +55 |
| 126 | 1.6 | 32.0 | 591.9614 | 1057.0739 | acceleration | +55 |
| 142 | 1.2 | 24.0 | 962.6837 | 1671.3259 | acceleration | -10 |
| 142 | 1.3 | 26.0 | 888.7383 | 1542.9484 | acceleration | -10 |
| 142 | 1.4 | 28.0 | 846.6763 | 1469.9241 | acceleration | -10 |
| 142 | 1.5 | 30.0 | 685.282 | 1189.7257 | acceleration | -10 |
| 142 | 1.6 | 32.0 | 544.0661 | 944.5592 | acceleration | -10 |
| 140 | 1.2 | 24.0 | 1039.3977 | 1804.5099 | static | -10 |
| 140 | 1.3 | 26.0 | 957.4538 | 1662.2462 | static | -10 |
| 140 | 1.4 | 28.0 | 868.163 | 1507.2274 | static | -10 |
| 140 | 1.5 | 30.0 | 722.5093 | 1254.3564 | static | -10 |
| 140 | 1.6 | 32.0 | 536.7567 | 931.8693 | static | -10 |

## Table 6.2: Cathode Active Material (FeS2) — Battery 44

| Build | V/Cell | V Battery | Ampere-Seconds | As/g FeS2 | Type | Temp |
| --- | --- | --- | --- | --- | --- | --- |
| 121 | 1.2 | 24.0 | 1034.929 | 782.2593 | static | -10 |
| 121 | 1.3 | 26.0 | 920.1289 | 695.4867 | static | -10 |
| 121 | 1.4 | 28.0 | 799.4474 | 604.2686 | static | -10 |
| 121 | 1.5 | 30.0 | 671.6689 | 507.6862 | static | -10 |
| 121 | 1.6 | 32.0 | 537.8419 | 406.532 | static | -10 |
| 146 | 1.2 | 24.0 | 1176.255 | 889.0816 | static | -10 |
| 146 | 1.3 | 26.0 | 1096.7926 | 829.0193 | static | -10 |
| 146 | 1.4 | 28.0 | 1013.8611 | 766.3349 | static | -10 |
| 146 | 1.5 | 30.0 | 880.5714 | 665.5868 | static | -10 |
| 146 | 1.6 | 32.0 | 628.9896 | 475.4268 | static | -10 |
| 123 | 1.2 | 24.0 | 1047.8731 | 792.0432 | static | -10 |
| 123 | 1.3 | 26.0 | 992.6273 | 750.2852 | static | -10 |
| 123 | 1.4 | 28.0 | 922.0028 | 696.9031 | static | -10 |
| 123 | 1.5 | 30.0 | 814.6308 | 615.7451 | static | -10 |
| 123 | 1.6 | 32.0 | 593.4171 | 448.539 | static | -10 |
| 117 | 1.2 | 24.0 | 1192.1597 | 921.5829 | static | +55 |
| 117 | 1.3 | 26.0 | 1021.9253 | 789.9855 | static | +55 |
| 117 | 1.4 | 28.0 | 917.5854 | 709.327 | static | +55 |
| 117 | 1.5 | 30.0 | 764.3768 | 590.8912 | static | +55 |
| 117 | 1.6 | 32.0 | 601.6418 | 465.0911 | static | +55 |
| 128 | 1.2 | 24.0 | 902.9985 | 682.5385 | acceleration | -10 |
| 128 | 1.3 | 26.0 | 850.8161 | 643.0961 | acceleration | -10 |
| 128 | 1.4 | 28.0 | 792.6398 | 599.1231 | acceleration | -10 |
| 128 | 1.5 | 30.0 | 731.1741 | 552.6637 | acceleration | -10 |
| 128 | 1.6 | 32.0 | 656.8669 | 496.498 | acceleration | -10 |
| 136 | 1.2 | 24.0 | 1140.8423 | 862.3147 | static | -10 |
| 136 | 1.3 | 26.0 | 1026.278 | 775.7203 | static | -10 |
| 136 | 1.4 | 28.0 | 897.3533 | 678.2716 | static | -10 |
| 136 | 1.5 | 30.0 | 721.0384 | 545.0026 | static | -10 |
| 136 | 1.6 | 32.0 | 501.4368 | 379.015 | static | -10 |
| 124 | 1.2 | 24.0 | 1236.2504 | 924.1612 | static | +55 |
| 124 | 1.3 | 26.0 | 1183.991 | 885.0946 | static | +55 |
| 124 | 1.4 | 28.0 | 1115.1081 | 833.601 | static | +55 |
| 124 | 1.5 | 30.0 | 996.0082 | 744.5677 | static | +55 |
| 124 | 1.6 | 32.0 | 720.9602 | 538.9551 | static | +55 |
| 120 | 1.2 | 24.0 | 1348.4582 | 1019.2428 | sttatic | +55 |
| 120 | 1.3 | 26.0 | 1252.9403 | 947.0448 | sttatic | +55 |
| 120 | 1.4 | 28.0 | 1081.0263 | 817.1023 | sttatic | +55 |
| 120 | 1.5 | 30.0 | 888.2178 | 671.3664 | sttatic | +55 |
| 120 | 1.6 | 32.0 | 629.7038 | 475.9666 | sttatic | +55 |
| 139 | 1.2 | 24.0 | 1034.1547 | 781.674 | static | +55 |
| 139 | 1.3 | 26.0 | 886.1616 | 669.8122 | static | +55 |
| 139 | 1.4 | 28.0 | 761.1055 | 575.2876 | static | +55 |
| 139 | 1.5 | 30.0 | 634.0775 | 479.2725 | static | +55 |
| 139 | 1.6 | 32.0 | 485.5575 | 367.0125 | static | +55 |
| 119 | 1.2 | 24.0 | 1144.4676 | 865.0549 | RV | +55 |
| 119 | 1.3 | 26.0 | 1088.9879 | 823.1201 | RV | +55 |
| 119 | 1.4 | 28.0 | 1008.0949 | 761.9765 | RV | +55 |
| 119 | 1.5 | 30.0 | 884.2091 | 668.3364 | RV | +55 |
| 119 | 1.6 | 32.0 | 683.9074 | 516.9368 | RV | +55 |
| 127 | 1.2 | 24.0 | 945.5325 | 714.6882 | RV | -10 |
| 127 | 1.3 | 26.0 | 893.5717 | 675.4132 | RV | -10 |
| 127 | 1.4 | 28.0 | 814.1897 | 615.4117 | RV | -10 |
| 127 | 1.5 | 30.0 | 709.5522 | 536.3206 | RV | -10 |
| 127 | 1.6 | 32.0 | 549.3842 | 415.2564 | RV | -10 |
| 113 | 1.2 | 24.0 | 1144.7112 | 874.9608 | static | -10 |
| 113 | 1.3 | 26.0 | 1011.2336 | 772.9371 | static | -10 |
| 113 | 1.4 | 28.0 | 784.8651 | 599.9122 | static | -10 |
| 113 | 1.5 | 30.0 | 584.943 | 447.1016 | static | -10 |
| 113 | 1.6 | 32.0 | 491.1146 | 375.3838 | static | -10 |
| 116 | 1.2 | 24.0 | 1035.9536 | 800.8299 | static | -10 |
| 116 | 1.3 | 26.0 | 954.1124 | 737.5637 | static | -10 |
| 116 | 1.4 | 28.0 | 884.3663 | 683.6474 | static | -10 |
| 116 | 1.5 | 30.0 | 734.6895 | 567.9418 | static | -10 |
| 116 | 1.6 | 32.0 | 515.3078 | 398.3517 | static | -10 |
| 138 | 1.2 | 24.0 | 998.4082 | 754.6547 | static | -10 |
| 138 | 1.3 | 26.0 | 902.681 | 682.2986 | static | -10 |
| 138 | 1.4 | 28.0 | 786.1354 | 594.2067 | static | -10 |
| 138 | 1.5 | 30.0 | 656.3404 | 496.1001 | static | -10 |
| 138 | 1.6 | 32.0 | 521.4444 | 394.1379 | static | -10 |
| 143 | 1.2 | 24.0 | 1216.1428 | 919.2311 | static | +55 |
| 143 | 1.3 | 26.0 | 1059.2012 | 800.6056 | static | +55 |
| 143 | 1.4 | 28.0 | 869.7995 | 657.4448 | static | +55 |
| 143 | 1.5 | 30.0 | 647.3766 | 489.3247 | static | +55 |
| 143 | 1.6 | 32.0 | 489.7894 | 370.2112 | static | +55 |
| 108 | 1.2 | 24.0 | 1282.3752 | 991.3228 | static | +55 |
| 108 | 1.3 | 26.0 | 1170.343 | 904.7178 | static | +55 |
| 108 | 1.4 | 28.0 | 981.7913 | 758.9605 | static | +55 |
| 108 | 1.5 | 30.0 | 769.1706 | 594.5969 | static | +55 |
| 108 | 1.6 | 32.0 | 571.9735 | 442.1564 | static | +55 |
| 126 | 1.2 | 24.0 | 958.5152 | 716.5397 | acceleration | +55 |
| 126 | 1.3 | 26.0 | 906.5281 | 677.6767 | acceleration | +55 |
| 126 | 1.4 | 28.0 | 865.6762 | 647.1378 | acceleration | +55 |
| 126 | 1.5 | 30.0 | 800.7958 | 598.6363 | acceleration | +55 |
| 126 | 1.6 | 32.0 | 591.9614 | 442.5218 | acceleration | +55 |
| 142 | 1.2 | 24.0 | 962.6837 | 727.6521 | acceleration | -10 |
| 142 | 1.3 | 26.0 | 888.7383 | 671.7599 | acceleration | -10 |
| 142 | 1.4 | 28.0 | 846.6763 | 639.967 | acceleration | -10 |
| 142 | 1.5 | 30.0 | 685.282 | 517.9758 | acceleration | -10 |
| 142 | 1.6 | 32.0 | 544.0661 | 411.2367 | acceleration | -10 |
| 140 | 1.2 | 24.0 | 1039.3977 | 785.637 | static | -10 |
| 140 | 1.3 | 26.0 | 957.4538 | 723.699 | static | -10 |
| 140 | 1.4 | 28.0 | 868.163 | 656.2079 | static | -10 |
| 140 | 1.5 | 30.0 | 722.5093 | 546.1144 | static | -10 |
| 140 | 1.6 | 32.0 | 536.7567 | 405.7118 | static | -10 |

---

## Table 9: Thermal Stack Calorific Value (Rules 7.1-9.7)

### Table 9: Build 108

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 13.2 |  |  |  |  |
| Cathode | 35.2 |  |  |  |  |
| Electrolyte | 25.6 |  |  |  |  |
| Heat Pellet-1 | 40 | 60.8 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 134.8 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 109

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 13.2 |  |  |  |  |
| Cathode | 35.2 |  |  |  |  |
| Electrolyte | 25.6 |  |  |  |  |
| Heat Pellet-1 | 40 | 60.8 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 134.8 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 112

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.0 |  |  |  |  |
| Cathode | 35.6 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 132.8 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 113

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.0 |  |  |  |  |
| Cathode | 35.6 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 132.8 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 116

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 13.2 |  |  |  |  |
| Cathode | 35.2 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 61.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 135.6 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 117

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 13.2 |  |  |  |  |
| Cathode | 35.2 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 61.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 135.6 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 119

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.6 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 134.0 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 120

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.6 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 134.0 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 121

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.6 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 134.0 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 123

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.0 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 133.2 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 124

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.0 |  |  |  |  |
| Cathode | 36.4 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.6 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 134.0 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 125

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.6 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 134.0 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 126

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.0 |  |  |  |  |
| Cathode | 36.4 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 133.6 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 127

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.0 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 133.2 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 128

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.0 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 133.2 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 135

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 27.2 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 18.4 |  |  |  |  |
| Total Stack Weight | 134.8 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 136

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 18.4 |  |  |  |  |
| Total Stack Weight | 133.6 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 137

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 54.0 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 18.4 |  |  |  |  |
| Total Stack Weight | 130.4 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 138

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 133.6 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 139

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 26 | 37.18 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 113.58 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 26 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 140

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 26 | 37.18 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 113.58 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 26 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 141

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 26 | 37.18 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 113.58 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 26 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 142

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 26 | 37.18 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 113.58 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 26 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 143

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 26 | 37.18 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 113.58 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 26 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 144

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 133.6 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Table 9: Build 146

| Row | Component | Wt/Pellet (g) | Qty | Total Wt (g) | CV (cal) |
| --- | --- | --- | --- | --- | --- |
| Anode | 14.4 |  |  |  |  |
| Cathode | 36.0 |  |  |  |  |
| Electrolyte | 26.0 |  |  |  |  |
| Heat Pellet-1 | 40 | 57.2 |  |  |  |
| Heat Pellet-1B | 0 | 0.0 |  |  |  |
| Current Collector | 29.6 |  |  |  |  |
| Total Stack Weight | 133.6 |  |  |  |  |
| Calorific value per gram of stack |  |  |  |  |  |
| Calorific value per gram per Ampere second |  |  |  |  |  |
| Total Calorific Value HP-1 | 40 |  |  |  |  |
| Total Calorific Value HP-1B | 0 |  |  |  |  |
| Total Calorific Value (Stack) |  |  |  |  |  |
| Calorific value per gram per second |  |  |  |  |  |

### Calorific Value Warnings

- ⚠️ Build 139: Rule 7.1.3 ERROR: Total Number of Cells (40) does not match the sum of Heat Pellet-1 Qty (26) and Heat Pellet-1B Qty (0).

- ⚠️ Build 140: Rule 7.1.3 ERROR: Total Number of Cells (40) does not match the sum of Heat Pellet-1 Qty (26) and Heat Pellet-1B Qty (0).

- ⚠️ Build 141: Rule 7.1.3 ERROR: Total Number of Cells (40) does not match the sum of Heat Pellet-1 Qty (26) and Heat Pellet-1B Qty (0).

- ⚠️ Build 142: Rule 7.1.3 ERROR: Total Number of Cells (40) does not match the sum of Heat Pellet-1 Qty (26) and Heat Pellet-1B Qty (0).

- ⚠️ Build 143: Rule 7.1.3 ERROR: Total Number of Cells (40) does not match the sum of Heat Pellet-1 Qty (26) and Heat Pellet-1B Qty (0).

---

## Table 12.0: Composite Design Data (Rule 12.0)

---

End of Comprehensive Report

Rules Applied

•  1.Definitions

•  Design- Data

•  Guidelines for determining discharge duration , activation time, on load voltage, open circuit.

•  The above parameters are calculated for different builds of one battery code.

•  There are two types of active material

•  To calculate discharge duration you must review the customer specifications and extract the following information:

•  Voltage time curves.